Ash free dry weight protocol

- Materials

  - Aluminum pans

  - Drying oven (60°C)

  - Desiccator

  - Muffle furnace (450°C)

  - Analytical balance (0.0001 g)

  - Lyophilized coral sample

- Protocol

  - If desiccant in desiccator has been sitting for a long time, or has been used for 1 month consistently, put in drying oven at 120°C for 4 hours using aluminum foil tray.

  - Obtain aluminum weigh pans to be used in AFDW determination

  - Label each pan with an ID number. May use a spatula to scrape a # into bottom of pan, or use pencil. Marker WILL burn off!

  - After labeling, stack the trays and place in the muffle furnace at 450°C for 4-6 h (or 500°C for 4-6).

  - After 4-6 h, turn off the furnace. If you need the furnace to cool quickly, you can also periodically open the door to the furnace if needed. This will take ~ 1 h.

  - After a cooldown period, remove from the muffle furnace using metal tongs and place in directly in a glass desiccator and transport to the scale room.

  - ** Take care after this point that pans are not touched without gloves on, and that pans are ALWAYS sat on burned aluminum foil on tabletops and in ovens/furnaces/scales ** All surfaces have residual carbon, even used gloves, so be very careful!!

  - Cover the bottom of the scale with a piece of burned tinfoil. Adjust-int on scale.

  - Record weight of burned pans in mg 4-decimal place scale (pan_weight_mg)

  - The scale sometimes takes a while to stabilize – make sure to close the glass and you’ll know it’s stabilized when the numbers turn from black to blue.

  - Store pans in desiccator until used.

  - Remove lyophilized samples from the freezer, wipe down tubes and place tubes in the drying oven at 60°C overnight to ensure samples are dry and remove any residual water from ice that may have accumulated on the outside of the tubes.

  - Let tubes cool and record weight of tube with lyophilized sample in it (total_weight_mg)

  - Subtract weight of empty tube (tube_weight_mg) to get total sample weight (sample_weight_mg)

  - Remove any amounts needed for other analyses (proteins, lipids, carbs, isotopes, etc.)

  - Weigh out ~10mg of lyophilized tissue into a pan. Record weight of pan + sample (dry_pan_mg) and calculate dry weight (dry_weight_mg).

  - After recording the weight of the burned pan + dry tissue, place in the muffle furnace and run at 450°C for 4-6hr.

    - Most literature uses a 6h burn period, but 4h is usually sufficient to burn off the well dispersed (high SA of sample in pans) residue.

    - The organic fraction will burn off at 450°C leaving only salt and inorganics behind.

    - The difference between the dry weight and burned weight is the organic fraction of coral biomass.

  - Record date_time_in_furnace. SET TIMER FOR 4-6HRS!!!! Make sure this is 4hrs + the time needed for the oven to reach 450°C (30min)

  - Come back 4-6hr later and turn off furnace. Record date_time_furnace_off. Let cool overnight.

    - Wait for pans to cool before removing them from the oven. If you need to remove them when they are still slightly warm, place in desiccator immediately and let them finish cooling there. If you remove the pans and weigh them when they are warm, moisture in the air will adhere to the pans, altering your weight.

  - Measure weight of burned pan + burned tissue (ash_pan_mg). Subtract pan weight to get the AFDW of the organic fraction (ash_weight_mg).

  - Calculate total AFDW for the fragment (total_afdw_mg) to get total tissue biomass by multiplying ash weight (ash_weight_mg) * dry weight (sample_weight_mg) and dividing by dry weight added (dry_weight_mg).

  - Calculate AFDW per cm2 of coral tissue (afdw_mg_cm2) by dividing AFDW (total_afdw_mg) by surface area (sa_cm2)

Table 1. Example calculations:

| A | B | C | D | E | F | G | H | I | J | K | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frag_ID | Total Weight mg | Tube Weight Mg | Sample Dry Weight mg | Burned Pan Weight Mg | Dry Pan weight Mg | Dry weight mg | Ash Pan Mg | Ash Weight mg | Total AFDW mg | SA | AFDW Mg Cm2 |
| Calc: |  |  | =B-C |  |  | =F-E |  | =H-G |  |  |  |
| Ex. 1 | 13394.49 | 13290.05 | 104.44 |  |  |  |  |  |  |  |  |